TC ELECTRONIC STUDIOKONEKT 48

INTRODUCTION

Studio Konnekt 48 is the member of the FireWire powered Konnekt audio interface family offering the most comprehensive feature set that the world has ever seen in an audio interface. 
It is the most complete product TC Electronic has ever made, and it represents a milestone to the computer recording world.

Studio Konnekt 48 offers a comprehensive array of professional features that make recording, mixing and monitoring a breeze. All essential recording tools are in one box including world-class DSP effects, a wealth of I/O options, speaker management and many other powerful tools to take your recordings to the next level.

The extensive I/O section features 4 IMPACT II™ preamps. With 8 line ins on the rear panel, the total amount of simultaneously streamable analog inputs is 12 as well as 12 analog outputs. Two outputs feature digitally controlled analog XLR connections for optimal connection with your main monitor speakers while the analog output section has 2 independent phones out.

The comprehensive digital I/O section features double ADAT 96 kHz SMUX, TOS, S/PDIF and Word Clock I/O and the digital part is powered by the second-to-none DICE Jet jitter elimination engine.

Studio Konnekt 48 comes with nothing short of world-class DSP effects, including the Fabrik C Studio, an easy-to-use channel strip and mastering tool that offers 4-band EQ, multiband compression and limiting based on algorithms from TC’s high-end System 6000. Also running on DSPs you'll find the Fabrik R Studio no compromise reverb based on TC Electronic Reverb 4000 technology.
It has a DSP powered 24/8 digital mixer with 48-bit double precision summing on all mix points. There is no compromise routing for all outputs and a wealth of insert points offering top-notch flexibility. 

Not only does Studio Konnekt 48 offer support for surround speaker systems, it does way more than that. The speaker management section is based on technology from the famous AIR™ speaker series, featuring individual speaker level and delay settings. The Bass Management feature gives you total control of your sub speaker and the speaker management offers instant control of up to 3 speaker sets.

Studio Kontrol Remote Unit

Studio Konnekt 48 comes with a cool, compact remote. With the Studio Kontrol remote at hand, you have access to all essential mixer parameters, including main level, mix levels, reverb level/decay time, speaker set selection, AUX sends and more. It has a built-in talkback microphone and a really cool light ring.

Anything else? Well, the cool factor of this remote is upped even further by the built-in guitar tuner.

FEATURES

The impressing feature list looks like this:

Extensive I/O section with IMPACT II™ preamps

- 4 IMPACT II™ mic preamps

- 12 analog inputs, 12 analog outputs simultaneously

- High resolution input meter

- 96 kHz ADAT (8 channels SMUX), word clock and S/PDIF

- 30/28 channels simultaneously to/from DAW

- Digitally controlled analog main XLR outputs

- Dual headphone outs with individual amplification, level control and source

24/8 channel digital mixer with 48-bit double precision summing

- Flexible 24/8 DSP mixer with total recall and DSP effects insert points

- No compromise routing for all outputs

- Full 48-bit double precision summing on all mix busses with 56-bit internal processing

- Talkback with selectable dim setting and listen back feature

- Aux send busses with flexible output routing

- World-class DSP effects

- 4 simultaneous 4-band EQ, multiband compression channel strip and mastering effects based on TC System 6000 algorithms

- No compromise built-in reverb based on TC Electronic Reverb 4000 technology

- All plug-in VST and AU compatible

- Intuitive guitar tuner

- ResFilter plug-in

- Assimilator Konnekt plug-in

- Hardware effects Integrator plug-in

Speaker management based on AIR™ technology

- Full featured bass management section based on AIR™ - Speaker Management Technology

- Full support for surround systems

- Individual speaker level and delay settings in 0.1 steps

- Selectable frequency crossover points

- Support for 3 speaker systems, individually configurable, selectable from the remote

Desktop remote control with integrated talkback microphone

- Full mixer at your hand

- Built-in talk back mic

- High resolution LED light ring

- Compact design

More

- DICE II JetPLL™ jitter elimination technology with Advanced Clock Recovery

- Expandable with all other Konnekt interfaces thru TC Near

# Extensive I/O section

The extensive I/O section features 4 IMPACT II™ preamps. 
With a very wide gain range, any type of microphone can be used with these great sounding mic preamps. Customized headroom ensures high quality recordings by treating very hot signal levels equal to low signal levels. Phantom power for condenser microphones goes without saying. The phantom power of Studio Konnekt 48 is compatible with all dynamic microphones and it is no problem using condenser microphones and dynamic microphones simultaneously. 

The four combo inputs on the front of Studio Konnekt 48 also feature inputs for high impedance instruments such as guitars or basses, and to get the best possible sound quality, there is a separate electronic circuit handling guitar signals. An auto detecting sensor detects whether you have attached an instrument and selects the appropriate circuit via the built-in relay switch.

With 8 TRS line inputs on the rear panel, the total amount of simultaneously streamable analog inputs is 12. The TRS inputs are balanced and the input sensitivity can be set to -10 or +4 dB. 

Analog and digital inputs can be monitored using the high resolution input display. 

On the rear panel, you will also find 8 TRS line outputs. These would do nicely with a surround speaker setup, typically 5.1 (left, right, center, LFE, left surround, right surround). Also, line outs would make sense for sending out separate mixes to external headphone amps, or even to an external mixing desk if needed. They are also perfect for integrating external analog hardware with your DAW based studio.

For your main monitor speakers we've provided special XLR connections. This balanced stereo output is analog, but at the same time it is digitally controlled. This combines the great analog sound with the flexibility and precision of digital control. 

The comprehensive digital I/O section features double ADAT (8 channels, 96 kHz SMUX), TOS, S/PDIF, MIDI and word clock I/O and the digital part is powered by the unmatched DICE Jet jitter elimination engine securing the best possible digital quality.
30 channels are streamed to and 28 channels are streamed from the DAW simultaneously.

The two headphone outputs on the front of Studio Konnekt 48 have individual amplification, level control and source. The left headphone output has its own D/A converter, while the right one uses the same D/A converter as line out 11/12. The flexible routing of Studio Konnekt 48 even allows you to route two independent signals to each side (left and right) of your headphones - if you're a DJ, you'll like this feature, because it allows you to have the main track in one side and the cue in the other - at the same time.

FireWire I/O
Studio Konnekt 48 is a 1394 FireWire based audio interface. One great advantage of this is that it is extremely easy to connect other Konnekt audio interfaces to your setup, thereby enhancing the number of I/Os and effect channels in your system. But not only is it easy to connect to your computer, the future proof network design also enables direct monitoring between devices.

An important part of the FireWire design of the Konnekt interfaces is based on a TC designed chip called DICE II™. The DICE II™ chip is the electronic part that makes the connection between all the inputs and outputs of Konnekt, but it does way more than that.

One important feature of the chip is the JET™ jitter elimination technology. JET™ is TC’s next generation of jitter elimination and sync handling, based on experience from TC flag ship System 6000, EQ Station and AIR™ speakers.

Jitter elimination
Caused by the variations in the timing of digital clocks controlling audio devices in the digital system, packets being streamed can arrive with varying delays. Jitter appears as audio degradation including reduced dynamic range and collapsed stereo fields.

With integrated JET™ technology Studio Konnekt 48 is able to perfectly align all digital signals from external digital sources, including the ones from FireWire. This means that all supported digital formats will be aligned to ensure maximal digital quality.

JetPLL™ makes sure that unwanted jitter is eliminated, and plays a significant part of why Studio Konnekt 48 will enhance the sound of your projects dramatically.

Advanced Clock Recovery
Sometimes users would like to synchronize the system to an external source. For this purpose, they use a house clock product to sync to. Studio Konnekt 48 can of course sync to an external source, using one of its digital ins, including word clock.

One of the problems with syncing to external sources is that the system is totally dependent on the sync device. If the connection is broke or the external device is turned of, the system will potentially stop playing. Some audio interfaces will automatically shift from external sync to internal sync, and this will keep audio playing, but at a different sample rate.
A unique feature in Studio Konnekt 48, called Advanced Clock Recovery, not only ensures that the system continues to play, but it also makes sure that it continues to play at the sample rate that it was syncing up to.

To learn more about JET, click here.

Thanks to the DICE II and the JET™ technology, the FireWire connections are used to their full potential. Not only does the DICE II ensure ultra stable audio streaming, it also gives you the potential of streaming a great number of simultaneous audio channels, without any loss of quality, and with an extremely accurate timing. And don’t forget that all of this can be done in a massive 192 kHz, 24-bit resolution.

# World-Class DSP

# Anybody familiar with TC, knows that our hardware products boast superior and award-winning sound quality. Studio Konnekt 48 effects are no exception! We are delighted to present you with “the sound of TC”. In fact, what you get is the capability and the sound quality of two full-blown 19” rack unit effects processors – built directly into Studio Konnekt 48.
The software package that comes with Studio Konnekt 48 contains a nice mix between plug-ins running on Studio Konnekt 48 DSPs and native plug-ins that run on your computer’s CPU.

# 24/8 channel digital mixer with 48-bit double precision summing

# Studio Konnekt 48 features a 24 in 8 out digital mixer with 48-bit double precision sum busses and 56-bit internal processing. The mixer runs on the DSPs of Studio Konnekt, and can therefore be used at all times; if your host application is not running or even if the computer is shut down. The mixer has insert points for the Fabrik plug-ins, also available in DSP. Each of the mixer’s outputs can be routed to any of the physical outputs.

With this mixer, you can skip your physical mixer in the studio; Studio Konnekt 48 offers all you need, and with the built-in DSP reverb, it even has a nice cue verb for the artist in the recording booth.

The mixer also features a comprehensive talkback feature with listen back, which allows you to communicate with the artist(s) in the recording booth. 

Studio Konnekt 48 comes with professional and innovative high quality TC effects. Running on the DSPs inside the audio interface you’ll find the Fabrik C Studio which is a 4-band EQ, multiband compressor and limiter. Fabrik C Studio works like a very nice channel strip and is very suited as a mastering tool and is based on technology from TC’s renowned System 6000. 

Also running on DSPs is the Fabrik R Studio, a TC Reverb 4000 based processor that includes 9 stunning reverb algorithms.

In MINT™ condition

The Fabrik C Studio and Fabrik R Studio plug-ins are both based on MINT™ – Meta Intuitive Navigation Technology – a novel approach to internal audio processing.

The design and programming philosophy gives you instant gratification and ensures that you won’t waste time tweaking a bunch of parameters to get professional results; simply get the sound you want by moving the MINT™ icons. It literally lets your ears back into audio production. Because that’s what it’s all about; listening, not tweaking. 

On top of that, we’re sure you’ll be amazed by the superior processing and sound quality that these MINT™ based effects have to offer.

Speaker management with bass management

Studio Konnekt 48 features support for surround systems at an unprecedented level for audio interfaces. The speaker management section is based on AIR™ technology, and it enables you to set individual speaker level and delay. 



This ensures perfect level alignment between speakers and if the physical nature of your control room is not perfectly suited for the speaker sets, the delay can be used to compensate for this.

Part of the speaker management section is the integrated bass management with selectable filters and frequency crossover points. The bass management system is designed to subtract the bass contents of the main channels and reproduce this by the use of a subwoofer. 

Typically, you’d check your mix on a number of different speakers. Therefore, most studios have at least a couple of speaker sets or three. Studio Konnekt 48 has support for 3 speaker sets and you can switch between the sets from the remote.

Intuitive guitar tuner
An intuitive guitar tuner is also part of the DSP effects included with Studio Konnekt 48. You can access the tuner from within the control panel. Choose between tuner modes, generate a reference tone for acoustic tuning and choose between a numbers of scales. 

An extra cool dimension is that you can also use the remote for tuning a guitar. In tuner mode, the lightring of the remote will show note deviation and by pressing the encoder button, you can generate the reference tone.

Native plug-in

The software package that comes with Studio Konnekt 48 contains a nice mix between plug-ins running on Studio Konnekt 48 DSPs and native plug-ins that run on your computer's CPU.

Natively running plug-ins included with Studio Konnekt 48 are:
ResFilter
Assimilator Konnekt
Integrator

To learn more about the included native plug-ins, please read on.

ResFilter
ResFilter is a resonance filter that offers ultra fat filtering effects. You can choose between 6, 12 and 24 dB slopes to get the filter sound you need, and the Drive option lets you add more juice to your sound.

Innovative, new communication technology offers communication between plug-ins. If you have more instances, you can have parameters from one instance control parameters of other instances. This will let you do cross-overs between tracks using cutoff to obtain the nice sound of filters blending in and out. If you insert a filter on each of your tracks, you can setup one of them to be "Master" to control all the others, assigning very powerful control options to just one button.
The number of instances you can run is only limited by the power of your CPU.

Assimilator Konnekt
Assimilator Konnekt is a native fingerprint EQ which is an extremely powerful equalization tool that “learns” the frequency curve of one piece of audio so it can be applied to another audio file – Ideal for analyzing and copying EQ curves of individual instruments, voices or reference mixes. It truly excels when applied to align the frequency response of different tracks recorded at different times compiled to appear on the same album.

Assimilator Konnekt derives from Assimilator – a very popular plug-in originally developed for PowerCore. It has been re-designed to run as a native plug-in.

Dweezil Zappa on Assimilator
Check out more amazing features of Assimilator on this video presentation – hosted by an all excited Dweezil Zappa and see what's gotten this world famous musician and producer so psyched up.

Integrator
The Integrator plug-in (VST and AU) makes integration of external hardware into your DAW environment seamless. It offers a “plug-in” audio routing representation of your external hardware: Reverb 4000, Finalizer, System 6000, etc.



Since Integrator operates like a normal VST/AU plug-in it can be inserted on any track, as a send effect or on your host application’s master output.

It works with up to four Konnekt audio interfaces at a time, so if you have more interfaces, simply select the one that the external hardware is attached to.

All plug-ins VST- and AU-compatible
All the plug-ins that come with Studio Konnekt 48 are VST and AU compatible. This means that you can use them with any VST and AU compatible host application, such as Logic, Garageband, Nuendo, Cubase, Ableton Live, Cakewalk etc. Even the DSP based effects are VST/AU compatible and can be used with those applications.

REMOTE

# Remote control

Desktop remote control with integrated talkback microphone With the optional Studio Kontrol remote, you have instant access to all important mixer parameters at all times. Use the encoder to control the selected function. The encoder controls two functions at a time - press it to shift between the two. The compact design of the remote makes it fit nicely beside the mouse in any computer based studio. At the upper section you find the input mix. This is where you control the level of the incoming signals - the mixer channels. This section is quite flexible, and from the control panel you decide which channels to control. Also the EFFECT button is flexible. By default it controls the reverb level and decay time, but it can also be assigned to control other parameters, for instance it can work as a midi controller. A talkback microphone is also part of the remote. The talkback microphone is enabled by pressing the TALK button. When in talkback mode, you can use the encoder to adjust the talkback level. In order to communicate most effectively, channels are dimmed to a selectable level, and to further optimize the talkback signal, you can insert a Fabrik C Studio for EQing, de-essing or compressing the voice. The talkback signal is routed to the DAW like any other input of Studio Konnekt 48, so you can make recordings directly from it if you want to make audio notes in your project or if you want to use it with other applications, such as Skype. The remote also gives you direct access to the guitar tuner. When in tuner mode, the lightring will show note deviation, and by pressing the encorder, you can generate a reference tone for tuning acoustic guitars. The connection between the Studio Kontrol remote and Studio Konnekt 48 is made by a single standard CAT5 cable. A neat solution that avoid lots of messy cables on your desk.

TECHS PSECS

# konnekt Comparison

# Tech Specs Digital inputs and Outputs. Connector (S/PDIF):; RCA Phono, 75 Ohm. Formats (S/PDIF and TosLink):; S/PDIF (24-bit), IEC 958, Pro-status bits. Connector (ADAT® or TosLink):; Optical Pipe. Format (ADAT®):; 8-ch. @ 48 kHz, 8-ch. SMUX @ 96 kHz. Connector (Word Clock IO):; BNC, 75 Ohm, 1 x Fs, 2.5 Vpp terminated, rising edge reference. FireWire (format):; IEEE 1394a, S400, IEC 61883. FireWire (channels):; 30 to and 28 from the DAW. Digital IO Engine:; TC DICE II, handling all IO formats. Clock and Jitter. Internal Sample Rates:; 44.1, 48, 88.2, 96, 176.4** and 192** kHz. Internal Clock Precision:; +/- 30 PPM. External Sample Rates:; 43 to 193 kHz, jitter rejection at all rates. Jitter Rejection Engine:; JET™ technology in TC DICE II. Jitter Rejection Filter:; <-3 dB @ 10 Hz, < -100 dB @ 600 Hz. DIO Interface Jitter:.